Plasmid Prep (classical protocol)

MATERIALS:

100% ethanol stored at -20C

70% ethanol stored at -20C

TEL solution:

10mM Tris-HCl pH 8.0

1mM EDTA

5g/L lyzozyme

NAS solution (prepare fresh):

0.2M NaOH

1% SDS

KAC solution (pH 4.8):

60mL 5M KOAc

28.5mL acetic acid

11.5mL dH2O

PROCEDURE:

- Resuspend the bacterial pellet in 200uL TEL solution.

- Incubate 5min on ice.

- Add 400uL NAS solution.

- Mix very well by inverting (don’t vortex!).

- Incubate 5min on ice.

- Add 300uL KAC solution.

- Invert the tube several times.

- Incubate 10min on ice.

- Spin for 4min at high speed.

- Take 600uL of super into a new tube.

- Add 1200uL of 100% ethanol stored at -20C.

- Incubate 10min on ice.

- Spin for 4min at high speed.

- Discard the super.

- Wash the pellet with 1mL of 70% ethanol stored at -20C.

- Spin for 2 min at high speed.

- Dry the DNA pellet at room temperature.

- Resuspend the pellet in 50uL 1X TE.

- Store at -20C.